3/3/2025

Newer exports IVIM contours are ‘autobinned.’ This means the data is stored differently, with two folders, one with the RTStruct, and the other with all 330 images, including the b0. The b0 in that folder is not in any specific spot, so we first need to find which 33 images are b0 in order for the pipeline to function.

- Download and unzip the autobinned file:

  - Inside:

    - A file with 330 images (including b0)

    - A file with one RTStruct

- Create a folder with the patient’s id, with two folders inside:

  - ‘DICOM’ which will contains all 330 images (including the b0)

  - ‘b0_Dicoms’ which will contain the 33 b0 images and the RTStruct

    - For ease, I bring in the two autobinned files here as well (so this folder will have 4 subfolders)

- Open MIM, and head to where the exported file is stored.

  - You can figure out which date the scan was pulled from by opening the autobinned file and reading the long title of one of the two folders (see an example below):

- Right click on the MRI with the session information

- Click "Show DICOM Information"

- At the top is a list of all images, scroll until you see sequence names "*ep_b0" (see image):

- Look at the SOP Instance IUD for the four numbers at the end. These correspond to the b0 images in the autobin folder. Keep track of these numbers.

- Inside the autobin file with all 330 images, search until you see the SOP Instance UID numbers (the last 4 digits on the image names) that correspond to the *ep_b0 images

- Copy these 33 images and paste them into the b0_Dicoms

- Rename the RTStruct to “RT_Struct.dcm” and move it into b0_Dicoms (b0_Dicoms now has 34 items inside)

- Copy all images into DICOM (330 items inside)

- Both original autobin files can now be deleted, leaving just DICOM and b0_Dicoms.

The following steps use code run in MATLAB R2024b. See here for used code: GitHub - miramliu/Pelvis_IVIM: IVIM and T1 of Pelvis Written for Dr. Marshall Lab · GitHub

- We need to sort the Dicom, creating a ‘DICOM_sorted’ folder
	SortIVIM_Pelvis(path_to_dicoms/PIPELINE_TEST_RUN/','DICOM');

- You can now follow the original pipeline document from step 5 onward: Pelvis_IVIM_PIPELINE_09242025.docx

If b0 were correctly isolated, the pipeline will work correctly. If done incorrectly, step 7 of the IVIM pipeline will fail due to matrices not being the correct size.